## Governing Differential Equation

- The Electrical System (KVL)

Applying Kirchhoff's Voltage Law (KVL) to the armature circuit loop:

Vt=VRt+VLt+et

Where:

- VR=itR (Voltage drop across armature resistance)

- VL=Lditdt (Voltage drop across armature inductance)

- et=Keωt=Kedθtdt (Back EMF produced by the rotating coil)

Substituting these values into the KVL equation yields Equation (1):

Lditdt+Rit=Vt-Kedθtdt

- The Mechanical System (Newton's 2nd Law)

The sum of torques acting on the rotor inertia must equal its angular acceleration:

T=Jαt=Jdωtdt=Jd2θtdt2

The acting torques are:

- Driving Motor Torque (Tm): Proportional to armature current, Tm=Ktit

- Opposing Friction Torque (Tf): Due to bearing viscous damping, Tf=bωt=bdθtdt

Setting up the balance equation:

Tdriving-Topposing=Jdωtdt

Ktit-bωt=Jdωtdt

Rearranging gives us Equation (2) in terms of angular velocity (ω):

Jdωtdt+bωt=Ktit

Or substituting ωt=dθtdt to express it in terms of angular position (θ):

Jd2θtdt2+bdθtdt=Ktit

## Transfer Function Development

Transforming to the Laplace Domain (s-Domain)

Assuming zero initial conditions (i0=0,θ0=0,θ0=0), we apply the Laplace Transform (ddt→s):

From the Electrical Equation (1):

LsIs+RIs=Vs-KesΘs

Ls+RIs=Vs-KesΘs --- (3)

From the Mechanical Equation (2) (using the position form):

Js2Θs+bsΘs=KtIs

sJs+bΘs=KtIs

Isolate current Is so we can substitute it out:

Is=sJs+bΘsKt --- (4)

Algebraic Substitution & Simplification

Substitute Equation (4) into Equation (3):

Ls+RsJs+bΘsKt=Vs-KesΘs

To eliminate the fraction, multiply the entire equation by Kt:

sJs+bLs+RΘs=KtVs-KtKesΘs

Move all terms involving Θs to the left-hand side:

sJs+bLs+RΘs+KtKesΘs=KtVs

Factor out sΘs on the left side:

Js+bLs+R+KtKesΘs=KtVs

Final Transfer Functions:

A. Angular Position Transfer Function: ΘsVs

Isolating Θs relative to input voltage Vs:

ΘsVs=KtsJs+bLs+R+KtKe

B. Angular Velocity Transfer Function: ΩsVs

Since angular velocity is the derivative of position, its Laplace relationship is Ωs=sΘs. Substituting this directly into our last factored step gives:

Js+bLs+R+KtKeΩs=KtVs

Dividing both sides yields the velocity transfer function:

ΩsVs=KtJs+bLs+R+KtKe

ΩsVs=KtJLs2+JR+bLs+bR+KtKe